LAB – 08

Name – Pradyut Laha							   Date – 16-11-2024

Vertical Seismic Profile (VSP) Processing and Corridor Stack Generation

Introduction:

Vertical Seismic Profiling (VSP) is a specialized seismic survey technique in which seismic receivers are placed inside a wellbore, and a seismic source is positioned on the surface to generate waves. In the case of Zero-Offset VSP (ZVSP), the source is placed as close as possible to the wellbore. This configuration provides critical information for understanding subsurface properties, including time-depth relationships, well-to-seismic calibration, and seismic velocity profiling.

In this report, we process a ZVSP dataset using a sequence of steps in SeismicUnix (SU) scripts. The primary objectives are to extract the time-depth relationship, determine average and RMS velocities, and generate a corridor stack for seismic interpretation. Each step involves preprocessing the raw data, separating wave components, applying deconvolution, and deriving key seismic attributes. This workflow ensures accurate calibration of well data with surface seismic surveys, enabling improved reservoir characterization and exploration decision-making.

The following sections detail the methodology and findings, including relevant plots and tables derived from the ZVSP analysis.

Interpretation:

Firstly, we need to use suswapbytes command to swap the dataset according to our environment. Next we will run the 0_2_utl_Display_ZVSP_RAW.sh file to visualize the raw VSP data.

Fig. 1 RAW VSP PROFILE

Next, we will run the 1_FBAutoPick.sh file to pick the first arrival. The default value of window_pick was given as 0.03, we have chosen the value as 0.015 to pick the first arrival perfectly.

| Fig. – 2 First Arrival picking using Window_size = 0.03 | Fig. – 3 First Arrival picking using Window_size = 0.015 |
| --- | --- |

Next we have to run 0_3_utl_CheckBPFandAmpRecovery.sh and 0_4_utl_FrequencyAnalysis.sh files to look into the suitable parameters that would help us to reduce the noises using F-X and F-K filters.

| Fig. – 4 BPF = 10,15,80,95 | Fig. – 5 BPF + RMS Normalization | Fig – 6 BPF + tpow-1.5 | Fig. – 7 BPF + RMS + Tpow |
| --- | --- | --- | --- |
| Fig. – 8 Picked Data | Fig. – 9 Amplitude Spectrum | Fig. – 10 Fk Filtering |  |

Now finally we have our desired parameters, so we can finally run the preprocessing shell script file by putting our desirable files. Finally we have our input and output files

| Fig. – 11 Input file data visualization | Fig. 12 – Corrected denoised and gain applied data |
| --- | --- |

Now we will run 3_Separation.sh file to separate the upgoing and downgoing events, and we can visualize the residual data after picking also.

| Fig. 13 – Z after Pre-processing | Fig. – 14 Downgoing rays | Fig. – 15 Residual - 1 |
| --- | --- | --- |

| Fig. 16 – Upgoing Reflections | Fig. 17 – Residual 2 |
| --- | --- |

Next we will run the 0_5_utl_CheckAutoCorrelation.sh file to check the auto-correlation graphs. The parameters chosen are used in the deconvolutional file.

Now we will run the 4_ApplyPEFDecon.sh file to get the deconvolved data. The outputs are shown below

| Fig. – 20 Upgoing before PEF Decon | Fig. 21 Upgoing after PEF Decon | Fig. 22 Upgoing after PEF Decon(ENHANCED) |
| --- | --- | --- |

Finally we can 5_CorridorStack.sh script to get out corridor stacked image.